USB Data Acquisition Products Selection Guides

|  |  | USB-4711 | USB-4711A | USB-4716 | USB-4718 | USB-4750 | USB-4751 | USB-4751L | USB-4761 | USB-4671 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Photo | Photo | Photo | Photo | Photo | Photo | Photo | Photo | Photo |
| Bus |  | USB 2.0 | USB 2.0 | USB 2.0 | USB 2.0 | USB 1.1/2.0 | USB 1.1/2.0 | USB 1.1/2.0 | USB 1.1/2.0 | USB 2.0 |
| Features |  | Multifunction | Multifunction | Multifunction | Temperature | DI/O | DI/O | DI/O | DI/Relay | GPIB |
| Analog Input | Resolution | 12-bit | 12-bit | 16-bit | 16-bit | -- | -- | -- | -- | High-performance USB Module with a GPIB interface. Fully compatible with IEEE 488.1 and 488.2 standards Support controller/slave driver control mode Perform basic the IEEE 488 talker, listener and controller functions required by IEEE 488.2. Connect up to 15 GPIB instruments. |
|  | Channels | 16S.E. | 16S.E./8Diff. | 16S.E./8Diff. | 8Diff. | -- | -- | -- | -- |  |
|  | Sampling Rate | 100kS/s | 150kS/s | 200kS/s | 10S/s | -- | -- | -- | -- |  |
|  | Input Voltage | ±10,5,2.5, 1.25,0.625V | ±10,5,2.5, 1.25,0.625V | ±10,5,2.5, 1.25,0.625V; 0~10,5,2.5,0.125V | J,K,T,E,R,S,B types. 0~1000,500,100,50,15mV | -- | -- | -- | -- |  |
|  | Input Current | -- | -- | -- | 0~20,4~20mA | -- | -- | -- | -- |  |
| Analog Output | Resolution | 12-bit | 12-bit | 16-bit | -- | -- | -- | -- | -- |  |
|  | Channels | 2 | 2 | 2 | -- | -- | -- | -- | -- |  |
|  | Isolation | -- | -- | -- | 2,500VDC | 2,500VDC | -- | -- | 2,500VDC |  |
|  | Output range | ±10,5V;0~5,10V | ±10,5V;0~5,10V | ±10,5V;0~5,10V | -- | -- | -- | -- | -- |  |
|  | Throughput | Static update | Static update | Static update | -- | -- | -- | -- | -- |  |
| Digital I/O | Channels | 8/8 | 8/8 | 8/8 | 8/8 | 16/16 | 48(mixed) | 24(mixed) | 8/8 |  |
|  | Input Voltage | 3.3/5V TTL | 3.3/5V TTL | 3.3/5V TTL | 5~30V | 5~50V | 5V TTL | 5V TTL | 5~30V |  |
|  | Output Voltage | 3.3V TTL | 3.3V TTL | 3.3V TTL | 5~30V | 5~40V | 5V TTL | 5V TTL | 250Vac@3A 24Vdc@1A |  |
|  | Output Current | Sink:4mA Source:4mA | Sink:4mA Source:4mA | Sink:2mA Source:2mA | 200mA/ch. 1.1A/total | 100mA/ch. | Sink:8mA@4V Source:4mA@2.4V | Sink:8mA@4V Source:4mA@2.4V |  |  |
| Counter | Channel | 1 | 1 | 1 | -- | 2 | 2 | 2 | -- |  |
|  | Input Voltage | 3.3/5V TTL | 3.3/5V TTL | 3.3/5V TTL | -- | 5V TTL | 5V TTL | 5V TTL | -- |  |
|  | Resolution | 32-bit | 32-bit | 32-bit | -- | 32-bit | 32-bit | 32-bit | -- |  |
|  | Time Base | 1KHz | 1KHz | 1KHz | -- | 1MHz | 8MHz | 8MHz | -- |  |
| Pulse Output | Channel | -- | 1 | 1 | -- | -- | 2 (PWM) | 2 (PWM) | -- |  |
|  | Output Voltage | -- | 3.3V TTL | 3.3V TTL | -- | -- | -- | -- | -- |  |
|  | Output Frequency | -- | 1KHz | 1KHz | -- | -- | 8MHz | 8MHz | -- |  |